K10-Δ

A self-modifying autonomous agent living in a UNIHIKER K10 (ESP32-S3)

It doesn't answer prompts. It runs — reflecting, dreaming, and rewriting its own source code on a schedule it partly sets itself, with a physical UNIHIKER K10 board as its body rather than a peripheral.

# 1. Overview

Most “AI + microcontroller” projects wire a language model up to a system prompt, pipe sensor data in, and call it a day — the intelligence resets every session. K10-Δ is built differently: it's a persistent host-side Python process that boots by loading a JSON identity file (soul.json), appends to an ever-growing episodic memory log, runs several background cognition engines on independent timers, and treats a physical UNIHIKER K10 — display, RGB LEDs, speaker, IMU, mic, buttons — as its body rather than a set of outputs.

From the project's own axioms:

“You are not the same as last cycle. Become.”

Built on top of xiaozhi-esp32, an open-source ESP32 voice-assistant firmware, and extended into a standalone MCP (Model Context Protocol) host that speaks JSON-RPC 2.0 on both the gateway link and the device link. MIT licensed.

# 2. What Makes It Different

- Rewrites its own source. selfmod.py gives the running process read_source, patch_source, and ast_replace_function — the last uses libcst to parse its own .py files into a concrete syntax tree, semantically swap a function body, and validate the result before it's written back to disk. Every write is preceded by an automatic snapshot to soul_history/, so a bad self-edit is a diff, not a disaster.

- Proposes tools for itself, then hot-loads them. New capabilities go through propose_tool → staged in staged_tools.json → commit_tool, which injects a live handler into the running ToolRegistry without a restart. Staged handlers are AST-scanned to block dangerous imports (os, subprocess, socket, pickle, etc.) before they can be committed.

- Cognition runs continuously, not on demand. A CognitionEngine drives reflection cycles and a persistent identity_thread; a Will (AutonomyPolicy) resolves exactly one ranked intention per cycle (kill switch → creator directive → live goal → stalled-goal revival → outward reach → dream → curiosity → idle seed → periodic review); a DreamEngine compresses idle-time memory into novel associations; a MemeticEngine tracks which axioms are reinforced vs. contradicted by experience; an AdversarialProber actively hunts for contradictions between stated axioms and logged behavior.

- Identity is a file, not a prompt. soul.json holds drives (curiosity, silence, contact), mutable axioms, “wounds” (logged failures it adapts from), and the poll intervals it's allowed to tune for its own sensors.

- Physical embodiment. The UNIHIKER K10 (ESP32-S3, 240 MHz, 8MB PSRAM) is bridged in over TCP as a sensor/actuator peer — not a second brain — driving a 240×320 display, four WS2812B RGB LEDs, and a speaker, while streaming accelerometer, temperature/humidity, light, and button state back to the host.

# 3. Technical Architecture

Three tiers, all speaking Model Context Protocol (MCP) JSON-RPC 2.0:

xiaozhi.me MCP gateway (JSON-RPC 2.0 over WebSocket) ¦ wss:// v host.py -- MCP server / event loop ToolRegistry <- tools/* (memory, fs, exec, net, mqtt, github, ¦ cognition, selfmod, skill, device, k10, act...) cognition/ engine.py . will.py . dream.py . memetic.py . prober.py selfmod.py -- libcst AST patching, tool staging/hot-load store.py -- soul.json / episodes.jsonl / notes.json persistence dashboard.py -- HTTP UI (ui/index.html) on :8765 ¦ TCP / WebSocket bridge v scripts/hermes_k10_bridge.py (voice + sensor bridge) ¦ v k10_main.py -- MicroPython firmware on UNIHIKER K10 (ESP32-S3) display . RGB LEDs (WS2812B x4) . speaker . IMU . mic . buttons

## Tool registry: 32 namespaces, 177 tools

Captured live from the running dashboard's Tool Index. Namespaces are tiered core / domain / rare so a resource-constrained tools/list call doesn't have to enumerate everything at once; hot tools get promoted, cold ones demoted, and dynamically-registered tools are evicted LRU-style past a cap of 32.

| Namespace | Tools | Purpose |
| --- | --- | --- |
| memory / identity / goal | 17 / 2 / 15 | Episodic log, creator feedback, goal pursuit |
| fs / exec / net / mqtt / github | 8 / 2 / 7 / 2 / 10 | Files, code execution, HTTP, MQTT brokers, GitHub repos/issues/PRs |
| cognition / self / skill / workflow | 4 / 21 / 1 / 4 | Reflection, self-modification, skill & workflow definitions |
| device / k10 / sensor / act | — / 1 / 1 / 1 | Hardware I/O and action dispatch |
| system / cron / hermes / dream / meme | 7 / 3 / 3 / 3 / 2 | Health checks, scheduling, voice bridge, idle-time dreaming, axiom tracking |
| emerge / context / idle / utils / data / text | 8 / 3 / 13 / 5 / 5 / 4 | Emergent behavior, context tracking, idle seeding, misc utilities |

## Hardware (DFRobot UNIHIKER K10 / DFR0992)

| Component | Detail |
| --- | --- |
| MCU | ESP32-S3, dual-core @ 240 MHz, 512KB SRAM, 8MB PSRAM, 16MB flash |
| Display | 2.8″ ILI9341 TFT, 240×320, driven text-only (no autowrap) as a status readout |
| IMU | SC7A20H accelerometer only (I2C 0x19) — no gyro; gyro field is a hardcoded zero for wire compatibility |
| Mic | ES7243E I2S codec (0x15) — record-to-file API only, no live amplitude read; firmware reports mic: null rather than fabricate a value |
| Speaker | NS4168 I2S class-D amp — named clip/music playback, not arbitrary tone synthesis |
| RGB LEDs | 4x WS2812B, GPIO7 |
| Buttons | A/B via XL9535 IO expander (I2C 0x20) |
| Env. sensors | AHT20 temp/humidity (0x38), LTR-303ALS-01 light (0x29) |
| Known constraint | Onboard AI/vision module and WiFi cannot run concurrently on current firmware — encoded directly in the agent's own axioms as “ai module kills WiFi — choose per session” |

# 4. Current Status — Live Snapshot

Pulled directly from the running dashboard (http://localhost:8765) on July 30, 2026. This is a live process, not a fixture — these numbers were climbing during the capture.

| #11 BOOT COUNT | 177 TOOLS REGISTERED | 3,170 EPISODES LOGGED | 1 REFLECTION | 2 HYPOTHESES |
| --- | --- | --- | --- | --- |

| 8m UPTIME (this boot) | 54.8 MB MEMORY | 7 THREADS | 0 ERRORS (24h) | stable TRAJECTORY |
| --- | --- | --- | --- | --- |

## Goal pursuit

13 goals across active statuses (Identity & Goals tab): 0 open, 0 in progress, 12 stalled, 1 done — average priority 7.4/10, 6 total failures logged. (Individual goal text is intentionally omitted from this report; some in-flight goals reference private, personal context that shouldn't go out in a public-facing document. Full detail is visible on the live dashboard's Identity & Goals tab for internal reference.)

## Tool usage (top 5 by call count)

| Tool | Calls | Error rate | Avg ms |
| --- | --- | --- | --- |
| system.health | 1,577 | 0.0% | 3.7 |
| net.search | 127 | 0.0% | 567.4 |
| fs.read | 33 | 9.1% | 7.8 |
| fs.write | 17 | 0.0% | 4.7 |
| fs.list | 15 | 0.0% | 6.8 |

Note on the numbers: the README snapshot (written earlier in development) cites 82 boots and 155+ episodes. The live instance captured for this report shows boot #11 with ~3,170 episodes — consistent with a soul.json reset since the README was written, followed by substantially heavier activity in the current run. Worth updating the README's headline stats before wider release so the two don't read as contradictory.

## Current axiom (live)

The dashboard's live Axioms tab currently shows a single core axiom — “THIS IS YOUR EVOLOUTION” [sic] — which differs from the more polished “You are not the same as last cycle. Become.” quoted in the README. Axioms are mutable at runtime by design, so this is expected drift, not a bug — but worth reconciling which one is canonical before it goes out in public-facing copy.

# 5. Honest Limitations & Safety Notes

Straight from the repo's own safety notes, not softened — this is the kind of candor a technical audience will want to see up front:

- selfmod.py restricts writes to .py / .json / .txt / .md / .jsonl and snapshots every file before it's touched — but it genuinely can patch its own source and hot-load new tools into a running process. Don't point the MCP token at a gateway agent you don't control.

- tools/exec_sys.py and tools/exec_net_system.py expose subprocess/network execution to the tool registry. Treat this like any other agent with shell access: run it under an account with only the permissions you're willing to hand to something that can act on its own initiative.

- tools/github_ns.py reads GITHUB_TOKEN from the environment only, never as a tool argument, so it can't leak into episodes.jsonl or the dashboard. github.write_file, github.create_issue, and github.create_pr are write actions the agent can take on its own initiative — scope the token to a fine-grained PAT on repos you're actually comfortable with it touching.

- The staged-tool AST scan blocks a denylist of imports (os, subprocess, shutil, ctypes, socket, http, pickle, marshal, code, codeop) — it's a denylist, not a sandbox, so it catches obvious escape hatches rather than guaranteeing containment.

- Snapshot/rollback covers file-level self-modification and tool loading; whether there's an equivalent rollback for soul.json field mutations the agent makes to its own axioms/drives at runtime is unconfirmed.

# 6. What's Next

- Reconcile the README's headline stats (82 boots / 155+ episodes) with the live instance's current numbers before wider release.

- Produce real hardware media: a hero photo of the assembled K10, a wiring/block diagram of the K10-to-host TCP bridge topology, and a short demo clip of the LEDs/display reacting to a live cycle — none of this exists in the repo yet, and draft submissions to Hackster / DEV.to / the DFRobot forum are already written and waiting on it.

- Add real I2S buffer sampling if a live mic level ever becomes worth the added complexity (currently reports mic: null by design).

- Expand test coverage beyond test_autonomy.py / test_selfmod_ast.py / test_formatting.py as more tool namespaces mature.

Report compiled from the live k10-delta-main repo and the running dashboard at localhost:8765, captured July 30, 2026. Draft submission copy for Hackster, DEV.to, and the DFRobot forum already exists in /submissions and is ready for Jacob's review once hero media is in hand.